Application for Permit to Drill

Kuvlum #2  OCS Y 0865 #1

Lease Block 672

date: August 1, 1993

will run MWD tools

100-110 ft water depths

70 18'36" 145 32'18.2"

UTM zone 6 x=554959.4 y=7800930

vertical well

Geological Data:

Proprietary geological and geophysical data has been submitted under a separate cover.  The Kuvlum prospect is formed by a down to the northeast growth fault truncating the flank of a structural nose which dips to the southwest.  The primary objective consists of stacked Oligocene fluvial to shelf sands expected to be encountered between 6000 and 10000 feet TVD ss.  Prominent seismic amplitudes, perhaps indicative of the presence of hydrocarbons, are associated with the prospect. Above the top of the Mid-Oligocene, major accumulations of hydrocarbons are not likely.  This prediction is based on the offset well data obtained from Arco's Kuvlum well #1 (OCS Y-0866), the onshore Pt.Thompson wells, Arco's Stinson Well #1, Unocal's Hammerhead Wells (OCS Y-0849 #1 and #2), SWEPI's Corona Well (OCS Y-0871 #1), Amoco's Galahad Well (OCS Y 1092 #1), Amoco's Belcher well (OCS Y-0917 #1) and ARCO's geological mapping and interpretation of the Kuvlum structure. Thus, for the Kuvlum Project Well #2 ARCO petitions that the top of the mid-Oligocene Formation should be recognized as the official 'Threshold Depth'. Below the mid-Oligocene is the secondary objective, the lower Oligocene Formation.

Seismic:

review seismic line BP89-16 sp 497 and CB90-500 sp 937

Oligocene is the primary objective

unit/age marker

Pliocene/Pleistocene   surface

top Miocene            900 ft md/rkb

top Oligocene          2640 ft md/rkb

top Primary Objective  6180 ft md/rkb (mid-Oligocene)